RESPA Monitoring System

New Customer / Device Setup Guide

NB Industries  |  Version 1.0  |  March 2026

# Overview

This document provides step-by-step instructions for setting up a new customer portal or adding a new device to an existing customer on the RESPA Monitoring System. The system uses Sorba Edge platform with Grafana dashboards for multi-tenant fleet monitoring.

Prerequisites:

- SSH access to Sorba server (192.168.1.206)

- Grafana admin credentials (sorba_admin / sbrQp10.)

- Senquip device provisioned and publishing via MQTT

- Equipment number and device type agreed with customer

# Section A: New Customer Setup

Follow these steps when onboarding an entirely new customer (e.g., BMA, Whitehaven).

## Step 1: Create Grafana Organisation

Each customer gets their own Grafana organisation for complete dashboard isolation.

curl -k -X POST https://192.168.1.206/dashboard/api/orgs \

-H "Content-Type: application/json" \

-u sorba_admin:'sbrQp10.' \

-d '{"name": "CUSTOMER_NAME"}'

Note the orgId returned (e.g., orgId: 5). You will need this for subsequent steps.

## Step 2: Add InfluxDB Datasource to New Org

Switch to the new org and add the InfluxDB datasource:

# Switch to new org (replace ORG_ID)

curl -k -X POST https://192.168.1.206/dashboard/api/user/using/ORG_ID \

-u sorba_admin:'sbrQp10.'

# Add InfluxDB datasource

curl -k -X POST https://192.168.1.206/dashboard/api/datasources \

-H "Content-Type: application/json" \

-u sorba_admin:'sbrQp10.' \

-d '{"name":"InfluxDB","type":"influxdb","url":"http://localhost:8086",

"database":"sorba_sde","user":"admin",

"secureJsonData":{"password":"sbrQp10."},

"access":"proxy","isDefault":true}'

## Step 3: Create Admin and Viewer Users

Create an admin user (for NBI management) and a viewer user (for customer access):

# Admin user

curl -k -X POST https://192.168.1.206/dashboard/api/admin/users \

-H "Content-Type: application/json" \

-u sorba_admin:'sbrQp10.' \

-d '{"name":"CUSTOMER Admin","email":"admin@customer.com",

"login":"customer_admin","password":"PASSWORD","OrgId":ORG_ID}'

# Viewer user (for customer access)

curl -k -X POST https://192.168.1.206/dashboard/api/admin/users \

-H "Content-Type: application/json" \

-u sorba_admin:'sbrQp10.' \

-d '{"name":"CUSTOMER Viewer","email":"viewer@customer.com",

"login":"customer_viewer","password":"PASSWORD","OrgId":ORG_ID}'

## Step 4: Create Fleet Overview Dashboard

Ask Claude to generate a Fleet Overview dashboard for the new customer. Provide:

- Customer name

- List of devices with Equipment Numbers and Senquip Device IDs

- Device types (truck, dragline, excavator, etc.)

Claude will generate the JSON and provide the curl commands to push it.

## Step 5: Set Home Dashboard

Set the Fleet Overview as the home dashboard for the org:

curl -k -X PUT https://192.168.1.206/dashboard/api/org/preferences \

-H "Content-Type: application/json" \

-u sorba_admin:'sbrQp10.' \

-d '{"homeDashboardUID":"FLEET_DASHBOARD_UID"}'

## Step 6: Switch Back to Main Org

curl -k -X POST https://192.168.1.206/dashboard/api/user/using/1 \

-u sorba_admin:'sbrQp10.'

# Section B: Adding a New Device

Follow these steps when adding a new Senquip device to an existing customer.

## Step 1: Configure Senquip Device

In the Senquip portal, configure the device MQTT settings:

| Setting | Value |
| --- | --- |
| Broker Address | 43.245.171.129 |
| Port | 8883 (TLS) |
| Client ID | DEVICE_ID (e.g., RR2EW78BF) |
| Username | sorba_sde |
| Topic | sorba/DEVICE_ID/data |
| TLS Certificate | NBI CA certificate |

IMPORTANT: Do NOT change the Senquip Device ID after setup. It forms the basis of the entire data pipeline.

## Step 2: Verify MQTT Data Flow

SSH to the Sorba server and verify data is arriving:

mosquitto_sub -h localhost -t "sorba/DEVICE_ID/data" -v

You should see JSON payloads arriving every 10 seconds.

## Step 3: Verify Sorba Auto-Discovery

Sorba should auto-create the asset in the workspace tree:

Instances > Respa > Assets > DEVICE_ID > data

If it does not appear, restart data-auto-sync:

sudo supervisorctl restart data-auto-sync

## Step 4: Configure Tag Aliases

In the Sorba Workspace, navigate to the device data tags and set aliases. The tag mapping table below shows the standard RESPA configuration:

| Tag | Alias | Description | Type |
| --- | --- | --- | --- |
| cp1 | cp1 | Differential Pressure (Pa) | Real |
| cp2 | cp2 | CO2 Concentration (ppm) | Real |
| cp3 | cp3 | Dust Concentration (ug/m3) | Real |
| cp8 | cp8 | Inlet Valve Status (Base64) | String |
| cp9 | cp9 | Outlet Valve Status (Base64) | String |
| cp10 | cp10 | Pressure Alarm (Base64) | String |
| cp11 | cp11 | CO2 Alarm (Base64) | String |
| cp12 | cp12 | Dust Alarm (Base64) | String |
| cp13 | cp13 | PFX Status (Base64) | String |
| cp14 | cp14 | CO2 Alarm Threshold (ppm) | Real |

## Step 5: Create Alarm Code Tags

Create six new tags for the decoded alarm codes. In Sorba Workspace, add these tags under the device data group:

| Tag Name | Type | Default | Data Logging |
| --- | --- | --- | --- |
| cp8_code | Real | 0 | Enabled, Raw Time, 10 sec |
| cp9_code | Real | 0 | Enabled, Raw Time, 10 sec |
| cp10_code | Real | 0 | Enabled, Raw Time, 10 sec |
| cp11_code | Real | 0 | Enabled, Raw Time, 10 sec |
| cp12_code | Real | 0 | Enabled, Raw Time, 10 sec |
| cp13_code | Real | 0 | Enabled, Raw Time, 10 sec |

## Step 6: Configure Alarm Decoder Script

In Sorba Script Engine, create or update the RESPA_Alarm_Decoder script to include the new device. The script reads Base64-encoded alarm strings from cp8-cp13 and writes decoded numeric codes to the _code tags.

Note the Redis tag IDs for the new device - these are needed in the script. Get them from:

redis-cli LRANGE rt_data:key_list 0 -1 | while read key; do

val=$(redis-cli GET "rt_data:$key")

echo "$key: $val"

done | grep "DEVICE_ID"

## Step 7: Set Up Alarm Sync Loop

Until Sorba data-auto-sync reliably writes _code tags, use the alarm sync workaround. Update /home/bradley/alarm_sync.sh to include the new device Redis tag IDs.

Alarm code mappings:

| Tag | Values | Meaning |
| --- | --- | --- |
| cp8_code | 1 = Connected, 0 = Disconnected | Inlet Valve |
| cp9_code | 1 = Connected, 0 = Disconnected | Outlet Valve |
| cp10_code | 0 = Normal, 1 = Low, 2 = High | Pressure Alarm |
| cp11_code | 0 = Normal, 1 = High CO2 | CO2 Alarm |
| cp12_code | 0 = Normal, 1 = High Dust | Dust Alarm |
| cp13_code | 1 = Opened, 0 = Closed | PFX Status |

## Step 8: Create Device Detail Dashboard

Ask Claude to create a RESPA device detail dashboard. Provide:

- Equipment Number (e.g., DRE01)

- Senquip Device ID (e.g., RR2EW78BF)

- Device Type (e.g., Dragline, Truck)

- Customer org ID

Claude will generate the dashboard JSON based on the proven v7 template and push it to the correct org.

## Step 9: Update Fleet Overview

The Fleet Overview table query needs a new row for each device. Claude will add a UNION query to the existing fleet dashboard and update the value mapping for the new device ID and Equipment Number.

## Step 10: Test and Verify

- Log in as the customer viewer user in an incognito browser

- Verify Fleet Overview shows the new device with live data

- Click device name to verify drill-down to detail dashboard

- Verify alarm status panels are showing correct states

- Verify GPS map shows device location

- Check trend data is recording over time

# Reference: Current System Configuration

## Existing Grafana Organisations

| Org ID | Name | Admin Login | Viewer | Status |
| --- | --- | --- | --- | --- |
| 1 | Main (Sorba) | sorba_admin | - | System |
| 2 | NBI | nbi_admin | nbi_viewer | Active |
| 3 | BMA | bma_admin | bma_viewer | Placeholder |
| 4 | Spare | spare_admin | spare_viewer | Placeholder |

## Existing Devices

| Eq # | Device ID | Customer | Type | Org ID |
| --- | --- | --- | --- | --- |
| DRE01 | RR2EW78BF | NBI | Dragline | 2 |

## Server Access

| Resource | Details |
| --- | --- |
| Sorba Server (LAN) | 192.168.1.206 |
| Sorba Server (Public) | 43.245.171.129:8080 |
| Grafana Login | https://192.168.1.206/dashboard/login |
| Grafana Public | https://43.245.171.129:8080/dashboard/login |
| InfluxDB | localhost:8086 (admin / sbrQp10.) |
| Redis | localhost:6379 (no auth) |
| Mosquitto MQTT | Port 1883 (plain) / 8883 (TLS) |
| Sorba Serial | 1762318931158 |

## ISO 23875 Alarm Thresholds

| Measurement | Green | Yellow | Red |
| --- | --- | --- | --- |
| CO2 (ppm) | 0 - 800 | 800 - 1000 | 1000+ |
| Dust (ug/m3) | 0 - 25 | 25 - 50 | 50+ |
| Pressure (Pa) | > 20 Pa | < 20 Pa | Negative |