SORBA RESPA MONITORING SYSTEM

System Reference & Configuration Guide

NB Industries — ICT & Systems Engineering

Last Updated: March 2026

# 1. System Architecture

Data flows from mining machinery through the following pipeline:

RESPA Advisor+ 2.0 → CAN Bus → Senquip ORB-C1-G (4G LTE) → SenquipConnect → MQTT (8883) → Sorba Edge Server → InfluxDB → Grafana Dashboards

| Component | Details |
| --- | --- |
| Senquip ORB Model | ORB-C1-G (4G LTE CAT-M1) |
| ORB Serial | RR2EW78BF |
| RESPA Model | Advisor+ 2.0 (Firmware v2.0.3) |
| Sorba Platform | Sorba Edge (SDE) — Trial License |
| Database | InfluxDB — sorba_sde |
| Measurement | Respa.RR2EW78BF.data |
| Dashboard | Grafana (Sorba-bundled) |
| CAN Protocol | J1939 — PGN 63649 (filter status) |

# 2. Server Configuration

## 2.1 Network

| Parameter | Value |
| --- | --- |
| Hostname | sorba |
| Internal IP | 192.168.1.206/23 |
| External IP (WARP) | 43.245.171.129 |
| CloudflareWARP Interface | 100.96.0.1/32 |
| DNS Domain | sorba.nbindustries.com.au (pending DNS A record) |
| Search Domain | nbi.local |

## 2.2 Service Ports

| Port | Service | Access |
| --- | --- | --- |
| 443 | Grafana Dashboard (via nginx) | External via port forward |
| 8883 | MQTT TLS (Mosquitto) | External via port forward |
| 1883 | MQTT (Mosquitto) | Internal only |
| 8080 | Grafana Direct | Localhost only (iptables) |
| 8086 | InfluxDB | Localhost only |
| 8096 | Sorba Core API | Localhost only |
| 8095 | Sorba Service | Localhost only |
| 3050 | Sorba Service | Localhost only |
| 8074 | Sorba Service | Localhost only |
| 5556 | Sorba Service | Localhost only |
| 3019 | Sorba Service | Localhost only |
| 9006 | Sorba Service | Localhost only |
| 8089 | Sorba Service | Localhost only |
| 20241 | Cloudflared Metrics | Localhost only |

## 2.3 Cloudflare Configuration

| Component | Details |
| --- | --- |
| WARP Client Tunnel | Sorba_VM — Device management & outbound traffic |
| Cloudflared Tunnel | sorba-office — Tunnel ID: 2651e5ab-81b2-4afe-9597-be431574a647 |
| Zero Trust Team | nbindustries.cloudflareaccess.com |
| WARP Split Tunnel Excludes | 198.41.192.0/24, 198.41.200.0/24 (Cloudflare edge IPs) |
| CIDR Route | 192.168.1.206/32 via sorba-office (for SSH via WARP) |

# 3. Login Credentials

## 3.1 Sorba Platform

| Service | Username | Password | URL / Notes |
| --- | --- | --- | --- |
| Sorba UI | bradley.houghton@nbindustries.com.au | Tr0nics@ | https://192.168.1.206 — Workspace, tags, data logging |

## 3.2 Grafana Dashboard Logins

| Org | Role | Username | Notes |
| --- | --- | --- | --- |
| NBI (Master) | Admin | nbi_admin | Access to ALL orgs & dashboards |
| Goonyella Riverside | NBI Admin | goonyella_nbi | Customer org only |
| Peak Downs | NBI Admin | peakdowns_nbi | Customer org only |
| Caval Ridge | NBI Admin | cavalridge_nbi | Customer org only |
| Saraji | NBI Admin | saraji_nbi | Customer org only |
| Hay Point | NBI Admin | haypoint_nbi | Customer org only |
| BMA | Admin | bma_admin | Customer org only |
| Spare | Admin | spare_admin | Customer org only |

## 3.3 System / API Logins

| Service | Username | Password | Notes |
| --- | --- | --- | --- |
| Grafana API Admin | sorba_admin | sbrQp10. | API & admin access |
| SSH | bradley | (SSH key / password) | ssh bradley@192.168.1.206 |
| InfluxDB | (no auth) |  | Database: sorba_sde |
| MQTT (Mosquitto) | See Sorba certs |  | /opt/sdc/certificate_manager/certs/NBI/ |

# 4. Grafana Organisations & Dashboards

| Org ID | Org Name | Dashboards |
| --- | --- | --- |
| 1 | Main Org. | RESPA Advisor - RR2EW78BF |
| 2 | NBI | RESPA Advisor, NBI Fleet Overview, NBI Master Operations |
| 3 | BMA | BMA Fleet Overview |
| 4 | Spare | Spare Fleet Overview |
| 5 | Goonyella Riverside | Goonyella Riverside Fleet Overview |
| 6 | Peak Downs | Peak Downs Fleet Overview |
| 7 | Caval Ridge | Caval Ridge Fleet Overview |
| 8 | Saraji | Saraji Fleet Overview |
| 9 | Hay Point | Hay Point Fleet Overview |

Note: nbi_admin is added to all orgs as Admin. Customer-specific logins are restricted to their org only.

# 5. RESPA Dashboard Panel Layout

## 5.1 Alarm Status Row

| Panel | ID | Data Source | Thresholds |
| --- | --- | --- | --- |
| Pressure Alarm | 61 | CAN.pressure_alarm | 0=Normal (green), 1+=Alarm (red) |
| CO2 Alarm | 62 | CAN.co2_alarm | 0=Normal (green), 1+=Alarm (red) |
| Dust Alarm | 63 | CAN.dust_alarm | 0=Normal (green), 1+=Alarm (red) |
| PFX Status | 64 | CAN.pfx_status | 0=Disconnected, 1=Connected |
| Inlet Valve | 65 | CAN.inlet_valve | Value mapping |
| Outlet Valve | 66 | CAN.outlet_valve | Value mapping |
| Tamper (Light) | 67 | DAS_Tag_Group.light | 0-49 lux=SEALED (green), 50+=TAMPER (red) |

## 5.2 Live Measurements

| Panel | ID | Data Source |
| --- | --- | --- |
| CO2 Concentration | 1 | CAN.co2_ppm (gauge) |
| Dust Concentration | 2 | CAN.dust_ugm3 (gauge) |
| Speed | 3 | DAS_Tag_Group.gps_speed (stat) |
| Temperature | 4 | DAS_Tag_Group.ambient (stat) |

## 5.3 Device Health

| Panel | ID | Data Source | Thresholds |
| --- | --- | --- | --- |
| Cellular Signal (RSSI) | 81 | DAS_Tag_Group.gsm_rssi | <-100 red, -100 to -85 orange, -85 to -70 yellow, >-70 green |
| Light Level (Tamper) | 82 | DAS_Tag_Group.light | <50 green, 50-200 yellow, >200 red |
| GPS Fix | 83 | DAS_Tag_Group.gps_fix | 0=No Fix (red), 1=2D (yellow), 2=GPS (green) |
| Satellites | 84 | DAS_Tag_Group.gps_nsat | <4 red, 4-8 yellow, >8 green |
| GPS Accuracy (HDOP) | 85 | DAS_Tag_Group.gps_hdop | <5 green, 5-10 yellow, >10 red |
| ORB Battery Voltage | 86 | DAS_Tag_Group.vsys | <3.3V red, 3.3-3.7V orange, >3.7V green |
| Cellular Signal Trend | 87 | DAS_Tag_Group.gsm_rssi | Timeseries chart |
| Light Level Trend | 88 | DAS_Tag_Group.light | Timeseries chart |

# 6. Decoded Data Fields

## 6.1 Base64 Status Fields (cp8-cp13)

| Field | Description | Example Decoded Value |
| --- | --- | --- |
| cp8 / cp8_code | PFX Connection Status | Disconnected (code 0) |
| cp9 / cp9_code | Pressure Alarm Text | Low Pressure Alarm (code 0) |
| cp10 / cp10_code | CO2/Dust Status | Normal (code 1) |
| cp11 / cp11_code | CO2/Dust Status | Normal (code 0) |
| cp12 / cp12_code | Valve State | Opened (code 0) |
| cp13 / cp13_code | Valve State | Opened (code 1) |

## 6.2 Key Numeric Fields

| Field | Description | Unit |
| --- | --- | --- |
| CAN.co2_ppm | CO2 concentration | ppm |
| CAN.dust_ugm3 | Dust concentration | µg/m³ |
| CAN.diff_pressure | Differential pressure | Pa or inWC |
| CAN.target_pressure | Target pressure setpoint | Pa or inWC |
| CAN.high_press_threshold | High pressure alarm threshold | Pa or inWC |
| CAN.low_press_threshold | Low pressure alarm threshold | Pa or inWC |
| CAN.high_co2_threshold | High CO2 alarm threshold | ppm |
| CAN.high_dust_threshold | High dust alarm threshold | µg/m³ |
| DAS_Tag_Group.vin | Vehicle supply voltage | V (12-28V typical) |
| DAS_Tag_Group.vsys | ORB internal LiPo battery | V (3.7-4.2V typical) |
| DAS_Tag_Group.vsys2 | ORB secondary voltage | V |
| DAS_Tag_Group.gsm_rssi | Cellular signal strength | dBm |
| DAS_Tag_Group.light | Light sensor (tamper) | lux |
| DAS_Tag_Group.ambient | Internal ORB temperature | °C |
| DAS_Tag_Group.gps_lat | GPS Latitude | ° |
| DAS_Tag_Group.gps_lon | GPS Longitude | ° |
| DAS_Tag_Group.gps_speed | GPS Speed | km/h |
| DAS_Tag_Group.gps_altitude | GPS Altitude | m |
| DAS_Tag_Group.gps_hdop | GPS Horizontal Dilution of Precision |  |
| DAS_Tag_Group.gps_nsat | GPS Satellite count |  |
| DAS_Tag_Group.gps_fix | GPS Fix status (0/1/2) |  |
| DAS_Tag_Group.gps_cog | GPS Course over ground | ° |

# 7. Useful Commands

## 7.1 Data Pipeline

| Task | Command |
| --- | --- |
| Latest data point | influx -execute 'SELECT * FROM "Respa.RR2EW78BF.data" ORDER BY time DESC LIMIT 1' -database 'sorba_sde' |
| Check light sensor | influx -execute 'SELECT last("DAS_Tag_Group.light") FROM "Respa.RR2EW78BF.data" WHERE time > now() - 5m' -database 'sorba_sde' |
| List all fields | influx -execute 'SHOW FIELD KEYS FROM "Respa.RR2EW78BF.data"' -database 'sorba_sde' |
| Check CAN fields | influx -execute 'SHOW FIELD KEYS FROM "Respa.RR2EW78BF.data"' -database 'sorba_sde' ¦ grep -i "CAN" |

## 7.2 Service Management

| Task | Command |
| --- | --- |
| Sorba services status | sudo supervisorctl status |
| Restart cloudflared | sudo systemctl restart cloudflared |
| WARP status | warp-cli status |
| WARP connect/disconnect | warp-cli connect / warp-cli disconnect |
| Cloudflared logs | sudo journalctl -u cloudflared --since "1 hour ago" --no-pager |
| Nginx reload | sudo nginx -t && sudo systemctl reload nginx |
| Disk usage | df -h / |
| Journal cleanup | sudo journalctl --vacuum-size=200M |

## 7.3 Grafana API

| Task | Command |
| --- | --- |
| List orgs | curl -sk -u sorba_admin:'sbrQp10.' https://192.168.1.206/dashboard/api/orgs |
| List dashboards (org 2) | curl -sk -u sorba_admin:'sbrQp10.' -H 'X-Grafana-Org-Id: 2' 'https://192.168.1.206/dashboard/api/search?query=' |
| Get dashboard JSON | curl -sk -u sorba_admin:'sbrQp10.' https://192.168.1.206/dashboard/api/dashboards/uid/respa-rr2ew78bf |
| List org users | curl -sk -u sorba_admin:'sbrQp10.' https://192.168.1.206/dashboard/api/orgs/2/users |

# 8. Firewall & Port Forwarding

## 8.1 Router Port Forwards

| External Port | Internal IP | Internal Port | Protocol | Service |
| --- | --- | --- | --- | --- |
| 443 | 192.168.1.206 | 443 | TCP | Sorba Dashboard (HTTPS) |
| 8883 | 192.168.1.206 | 8883 | TCP | MQTT TLS (Senquip) |

## 8.2 iptables Rules (Localhost Only)

The following ports are restricted to localhost only via iptables: 8096, 8095, 3050, 8080, 8074, 5556, 3019, 9006, 8089. External access to these services is through nginx reverse proxy on port 443.

# 9. Outstanding Items

| Item | Priority | Status |
| --- | --- | --- |
| Sorba production license | High | Contact Sorba for permanent license (trial resets every 2 hours) |
| DNS A record for sorba.nbindustries.com.au | High | Request sent to DNS contractor → point to 43.245.171.129 |
| PGN 63649 filter data decoding | Medium | Waiting for cab test with filter installed |
| WARP SSH from home | Medium | CIDR route set — install WARP client on home PC |
| Nginx hostname update | Low | Add sorba.nbindustries.com.au to nginx server_name after DNS |
| Disk space investigation | Low | 25GB in /usr/share (vision_service ML venvs) — investigate if removable |
| SSH brute force mitigation | Low | Frequent kex_exchange_identification errors from external IPs |

# 10. MQTT Certificate Paths

| File | Path |
| --- | --- |
| CA Certificate | /opt/sdc/certificate_manager/certs/NBI/ca.crt |
| CA Key | /opt/sdc/certificate_manager/certs/NBI/ca.key |
| Client Certificate | /opt/sdc/certificate_manager/certs/NBI/client.crt |
| Client CSR | /opt/sdc/certificate_manager/certs/NBI/client.csr |
| Client Key | /opt/sdc/certificate_manager/certs/NBI/client.key |